Methodology: Throughput and Energy Efficiency Formulas

This section presents the closed-form expressions used throughout the paper to derive execution time, throughput, energy, and energy-efficiency metrics for the three RISC-V based SoC configurations under comparison (RISP, RV32I, and Ibex). All three cores are clocked at f = 40 MHz (T = 25 ns/cycle); the measured Total On-Chip Power for each SoC is P = 0.407 W (RISP), P = 0.438 W (RV32I), and P = 0.446 W (Ibex). Every throughput and energy figure reported in the paper is derived solely from these two quantities (T and P) together with the cycle count N_cycles obtained from RTL simulation for each workload.

## 1. Execution Time

The execution time of a workload is obtained directly from the measured cycle count and the clock period:

t = Ncycles × T	(1)

## 2. Throughput

Throughput, expressed in Mb/s (megabits per second), normalizes the amount of data processed, S_block, by the execution time:

Throughput = Sblockt = SblockNcycles × T	(2)

S_block is 128 bit for every AES-128 mode (ECB, CBC, CTR, CFB). For SHA3-256, S_block is the actual input-message length in bits (message bytes × 8) — not the 1088-bit algorithmic rate.

## 3. Energy Consumption

Energy is obtained from the measured power and the execution time; energy per bit normalizes this figure by the amount of data processed:

Energy = P × t = P × Ncycles × T	(3)

Energy/bit = EnergySblock	(4)

Energy is reported in nJ for AES-128 and SHA3-256 (short execution times) and in µJ for RSA-2048 (substantially longer execution time).

## 4. Throughput per Watt

Throughput per Watt (Mb/s/W) is the primary energy-efficiency figure of merit used for cross-core comparison:

Mb/s/W = Throughput (Mb/s)P (W)	(5)

This quantity is always the reciprocal of Energy/bit up to a factor of 1000, providing a convenient self-consistency check:

Mb/s/W ≈ 1000nJ/bit	(6)

## 5. RSA-2048-Specific Formulas

RSA-2048 verification is a single modular-exponentiation operation rather than a continuous data stream, so throughput is expressed in verify/s instead of Mb/s:

t = Ncycles × T	(7)

Energy = P × Ncycles × T	(8)

Throughput = 1t  (verify/s)	(9)

Throughput/W = 1tP  (verify/s/W)	(10)

## 6. Cross-Check Invariant (AES-128)

For AES-128, the product of cycle count and throughput is a core-independent and mode-independent constant, providing a quick sanity check on reported figures:

Ncycles × Throughput (Mb/s) = Sblock (bit) × f (MHz) = 128 × 40 = 5120	(11)

Any reported AES-128 (N_cycles, Throughput) pair whose product deviates from 5120 indicates an error in either the cycle count or the throughput calculation.

## Nomenclature

| Symbol | Definition |
| --- | --- |
| T | Clock period, T = 1/f = 25 ns at f = 40 MHz. |
| P | Total on-chip power consumption (W), measured per SoC: RISP = 0.407 W, RV32I = 0.438 W, Ibex = 0.446 W. |
| N_cycles | Number of clock cycles required to complete a given operation, measured from RTL simulation. |
| S_block | Size of the data block processed per operation, in bits (fixed at 128 bit for AES-128; equal to the input message length × 8 for SHA3-256). |
| t | Execution time, t = N_cycles × T. |

## Illustrative Example — RISP Core (f = 40 MHz)

Table I applies Eqs. (1)–(6) to the AES-128 and SHA3-256 workloads measured on the RISP core (P = 0.407 W).

TABLE I

AES-128 / SHA3-256 THROUGHPUT AND ENERGY — RISP CORE

| Workload | N_cycles | S_block (bit) | t | Throughput | Energy | Energy/bit | Mb/s/W |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AES-128 ECB | 88 | 128 | 2.2 µs | 58.18 Mb/s | 895.4 nJ | 7.00 nJ/bit | 142.95 |
| AES-128 non-ECB (CBC/CTR/CFB) | 124 | 128 | 3.1 µs | 41.29 Mb/s | 1,261.7 nJ | 9.85 nJ/bit | 101.45 |
| SHA3-256 (1 block, 128 B) | 492 | 1,024 | 12.3 µs | 83.25 Mb/s | 5,006.1 nJ | 4.89 nJ/bit | 204.55 |
| SHA3-256 (4 block, 536 B) | 1,771 | 4,288 | 44.275 µs | 96.85 Mb/s | 18,020 nJ | 4.20 nJ/bit | 237.96 |
| SHA3-256 (16 block, 2,168 B) | 6,871 | 17,344 | 171.775 µs | 100.97 Mb/s | 69,912 nJ | 4.03 nJ/bit | 248.08 |

Table II applies Eqs. (7)–(10) to the RSA-2048 verify operation measured on the RISP core.

TABLE II

RSA-2048 VERIFY — THROUGHPUT AND ENERGY — RISP CORE

| Workload | N_cycles | t | Energy | Throughput | Throughput/W |
| --- | --- | --- | --- | --- | --- |
| RSA-2048 Verify | 320,972 | 8.0243 ms | 3,265.89 µJ | 124.62 verify/s | 306.19 verify/s/W |